REEV CONCEPT

Range-Extended Electric Vehicle

India-Focused Architecture Concept  ·  INGLO Platform Derivation

Concept document  ·  August 2026  ·  Pre-feasibility stage  ·  Not for regulatory submission

# 1.  Executive Summary

This document introduces a Range-Extended Electric Vehicle (REEV) concept designed specifically for Indian driving and infrastructure conditions. The concept is derived from the Mahindra XEV 9S INGLO BEV platform and retains its 400 V architecture, rear electric drive unit, and proven body structure. The combustion engine never drives the wheels — traction is always electric.

The core proposition: a 40/50 kWh battery pack handles daily city commuting without the engine starting, while a dedicated 105 kW petrol generator provides unlimited range on any highway in India. The buyer gets a city-capable EV that never needs a charger on a long trip.

| Item | RWD Variant | AWD Variant |
| --- | --- | --- |
| Battery (usable) | 40 kWh | 50 kWh |
| Peak battery power | 187 kW (4.67C) | 263 kW (5.26C) |
| Traction architecture | Rear EDU — always electric | Dual EDU — always electric |
| Generator output | 105 kW HV-DC peak | 105 kW HV-DC peak |
| Fuel tank | 42 L petrol | 42 L petrol |
| Programme kerb target | 2,300 kg | 2,500 kg |
| MIDC EV range (calibrated) | ~340 km | ~408 km |
| Real-world EV range | ~200–260 km | ~240–310 km |
| Total range | Unlimited — petrol backstop | Unlimited — petrol backstop |
| HV bus voltage | 400–465 V nominal | 400–465 V nominal |

# 2.  The Problem This Solves

Three distinct customer anxieties prevent BEV adoption in India that this architecture resolves simultaneously:

Range and charging anxiety  Charging infrastructure is too sparse for highway use. Less than 20% of Indian highways have DC fast chargers at the density required for a pure BEV. The REEV's generator eliminates this concern entirely — petrol is available at over 80,000 fuel stations across India.

Petrol running cost  Running on petrol at 8–10 km/l, a premium SUV costs ₹10–13/km in city conditions. The same REEV route on grid electricity costs approximately ₹2/km — one-fifth of the petrol bill for the identical commute.

Diesel maintenance and 10-year ban  BS6-II diesel SUVs carry DPF replacement costs of ₹60,000–₹1,20,000 and AdBlue consumption of ₹2,000–₹4,000 per service. SC and NGT orders prohibit diesel vehicles older than 10 years in Delhi NCR and expanding metro zones.

Strong hybrid — cost without the escape route  Strong hybrids at 40% GST carry the same tax as a full ICE vehicle. With the proposed 18% REEV classification, this concept would retail at a lower on-road price than an equivalent strong hybrid while delivering genuine daily EV range.

# 3.  Platform Derivation — Mahindra INGLO

The concept is built on the XEV 9S INGLO skateboard. The INGLO platform carries most of its investment in the body, suspension, electronics, and HV architecture. A REEV derivative touches only two areas: the energy storage and the space behind the firewall.

| System | INGLO / XEV 9S baseline | REEV modification |
| --- | --- | --- |
| Body structure (BIW) | Retained | Purpose-built REEV floor for shorter pack |
| Front suspension | Retained | Unchanged |
| Rear EDU (traction motor) | Retained — drives wheels | Retained — still drives wheels in RWD |
| AWD front EDU | Present in AWD variant | Retained; housing carried in RWD body (structural) |
| 400 V HV architecture | Retained | Same 108-series cell string, same bus |
| 79 kWh Blade 1.0 LFP pack | Replaced | 40/50 kWh Short Blade 2.0 high-power phosphate |
| Engine / generator / fuel | Not present | Added: 1.5T TGDi + PMSG + 42 L tank |
| Exhaust / after-treatment | Not present | Added |

# 4.  Battery Pack — Why 40/50 kWh, and Why These Cells

The pack sizes are not chosen to maximise range. They are set by peak power demand at a healthy C-rate. The motor calibration requires 187 kW DC from the RWD pack and 263 kW from the AWD pack.

| Variant | Usable capacity | Peak power demand | C-rate | Cell Ah (108s 1p) | Bus voltage |
| --- | --- | --- | --- | --- | --- |
| RWD | 40 kWh | 187 kW | 4.67C | ~97.5 Ah | ~410 V nominal |
| AWD | 50 kWh | 263 kW | 5.26C | ~121.8 Ah | ~410 V nominal |
| XEV 9S BEV (reference) | 79 kWh | 187 kW | 2.37C | ~192 Ah | ~410 V nominal |

The 79 kWh INGLO Blade 1.0 LFP cell runs at a comfortable 2.37C. At 40 kWh the C-rate nearly doubles — the Blade 1.0 was not characterised for this duty. The BYD Short Blade 2.0 high-power phosphate variant is designed for PHEV/REEV shallow cycling, high pulse power, and repeated generator charging.

The 40/50 kWh pair is the smallest round-kWh split at which both variants stay at or below 5.3C with minimal asymmetry. Both use a 108-series 1-parallel string, maintaining the 400 V bus without any step-up converter. Certified MIDC EV range — ~340 km RWD, ~408 km AWD — comfortably exceeds the proposed 270 km policy threshold. This headroom is a consequence of correct pack sizing, not a design target.

# 5.  Engine-Generator System

The 1.5-litre turbocharged petrol engine (mStallion TGDi class, ~105 kW crank output) drives a PMSG through a single fixed-ratio coupling. The rectified HV-DC output feeds directly onto the 400 V bus. The engine has no mechanical path to the wheels at any point.

| Band | HV-DC output | Engine RPM | Brake thermal efficiency | Typical use |
| --- | --- | --- | --- | --- |
| 1 | 30 kW | 1,500 | 38.5–40.0% | Turtle recovery / minimum sustaining |
| 2 | 40 kW | 1,800 | 37.5–39.0% | Light highway |
| 3 | 55 kW | 2,300 | 36.5–38.0% | Normal highway |
| 4 | 70 kW | 2,800 | 35.0–37.0% | Sustained high load |
| 5 | 90 kW | 3,500 | 33.5–35.5% | High demand / hills |
| 6 | 105 kW | 4,000 | 31.5–34.0% | Maximum output |

# 6.  Energy Management — Control Law Summary

| State | Condition | Generator | Battery role |
| --- | --- | --- | --- |
| S1 — Pure EV | SoC above mode start boundary | Off | Supplies all traction and auxiliaries |
| S2 — Battery protection | SoC healthy but capability limited | Off | Torque reduced; no generator start for acceleration |
| S3 — Charge sustaining | SoC at mode start boundary | Running in smallest sufficient band | Covers transients; surplus recharges battery |
| S4 — Generator limited | Battery cannot sustain traction deficit safely | Running; covers auxiliaries + traction | Discharge limit zero; accepts generator surplus only |
| S5 — Turtle | Battery at Turtle floor, no fuel | Off | Emergency mobility; 60 km/h ceiling |
| S6 — Refuel recovery | Fuel confirmed from S5 | Restarting at 30–40 kW | Energy rebuilt before performance restored |

Three driver-selectable modes — Economy (15%/30%), Sport (20%/35%), Hills/Terrain (20%/35% with longer downshift dwell) — adjust generator boundaries and ramp rates. Sport demotes to Economy at 9.5% SoC. Hills/Terrain holds its band-hunting character down to 8%.

# 7.  Mass Budget

XEV 9S reference: 2,240 kg with 79 kWh pack. All values are planning ranges, not weighed results:

| Item | RWD Δ kg | AWD Δ kg |
| --- | --- | --- |
| Remove: 79 kWh Blade 1.0 LFP pack | −550 to −560 | −550 to −560 |
| Add: 40 kWh Short Blade 2.0 RWD pack | +240 to +270 | — |
| Add: 50 kWh Short Blade 2.0 AWD pack | — | +295 to +330 |
| Add: Engine + fluids + PMSG + coupling + rectifier + intake | +230 to +290 | +230 to +290 |
| Add: Exhaust + after-treatment + cooling + NVH shields | +54 to +85 | +54 to +85 |
| Add: 42 L fuel tank + pump + lines + vapour system | +40 to +50 | +40 to +50 |
| Add: Thermal / structural / NVH integration | +15 to +30 | +15 to +30 |
| Add: AWD front 70 kW EDU + subframe + HV hardware | — | +145 to +195 |
| BIW / floor optimisation opportunity | −20 to −70 | −20 to −70 |
| Planning range vs XEV 9S | ~2,170 to ~2,330 kg | ~2,360 to ~2,545 kg |
| Programme kerb target | 2,300 kg | 2,500 kg |

# 8.  Running Costs vs Alternatives

Road-load model at 2,300 kg RWD, 100 km/h highway, normal load. REEV assumes 95% grid charging in city. Fuel prices: petrol ₹104/L, diesel ₹92/L, electricity ₹8/kWh.

| Vehicle | City ₹/km | Highway ₹/km | 10-yr fuel cost (10,000 km/yr) |
| --- | --- | --- | --- |
| REEV RWD — city on grid | ~₹2.0 | ~₹4.5 (generator) | ~₹3.3 lakh |
| XEV 9S BEV (79 kWh) | ~₹2.0 | ~₹3.8 | ~₹3.0 lakh |
| 2.2 mHawk diesel AT | ~₹8.2 | ~₹6.2 | ~₹7.2 lakh |
| 2.0 mStallion petrol AT | ~₹13.7 | ~₹10.6 | ~₹12.2 lakh |
| Illustrative 185 hp HEV | ~₹5.6 | ~₹6.1 | ~₹5.9 lakh |

# 9.  Policy Proposal — GST Classification

A capability-based GST class defined by measurable, independently verifiable criteria. Current GST on an over-4-metre petrol SUV above 1,500 cc is 40%:

| Class | Proposed GST | Generator displacement | Certified MIDC all-electric range |
| --- | --- | --- | --- |
| Above 4 metres | 18% | Above 1,000 cc and below 1,500 cc | At least 270 km |
| Sub-4 metres | 9% | 500 cc to below 1,000 cc | At least 210 km |
| At or above 1,500 cc | 40% | 1,500 cc and above | No preferential rate |

The fiscal case: GST forgone is ~₹0.86 lakh net delta per vehicle (₹4.73L forgone minus ₹3.87L still collected at 18%). Crude import saving recovers that net delta in under 4 years for a typical user. At 1 lakh vehicles the ₹860 crore net delta is recovered from ₹286 crore/year in avoided crude imports in approximately 3 years.

# 10.  Competitive Context

| Vehicle | Architecture | Pack | Key distinction vs this concept |
| --- | --- | --- | --- |
| Li Auto L6 (China) | Series REEV | 52.3 kWh | Premium estate; no mechanical wheel path |
| Leapmotor C10 REEV (Europe) | Series REEV | 28.4 kWh | B-segment; smaller pack; lower price point |
| Aito M7 (China) | Series REEV (Huawei Driveone) | ~40 kWh | Premium SUV; HarmonyOS integration |
| MG ADAPT (India, Jul 2026) | DHT — parallel hybrid + REEV mode | 20.5 kWh PHEV | Multi-mode; REEV deprioritised; 400V capable |
| This concept | Dedicated series REEV only | 40 / 50 kWh | INGLO derivation; India-specific control law; always-electric wheels |

# 11.  Open Questions and Next Steps

Pack and cell  Cell selection, pack topology, voltage and current maps, pulse durability, thermal analysis — BYD Short Blade 2.0 is the candidate but supplier datasheet data is required.

Engine-generator  Engine-generator dyno maps for the specific mStallion TGDi + PMSG unit. Band BTE values are provisional abstractions.

Road-load constants  Coast-down testing to confirm CdA, Crr, and kerb mass on a production-representative vehicle.

Regenerative braking  Brake-by-wire hardware design, ABS/ESC integration, and homologation for the 70% regen recovery assumption.

ARAI certification  ARAI OVC-HEV procedure agreement — test family, variant selection, and break-off criterion require regulatory alignment.

This is an independent, engineer-led pre-feasibility concept developed from publicly available information and first-principles calculations. It is not a production specification. Every effort has been made to keep the architecture, performance and cost assumptions grounded, but they require validation through packaging studies, supplier data, simulation, dynamometer testing and homologation. No vehicle has been built, tested, or homologated.